Washington Park Design Plan Overview

Site 1 – https://maps.app.goo.gl/jj8Epm35hFG3tmbi9

Located at the park along riverside drive between Pine St and Walnut St. This proposed site will install a living shoreline with Marine sand fill and native plantings along the eroded shoreline between 2 groins located at the piles from the abandoned dock. Note that there will be no planning in Beach Access Zones. These area will be open for access.

Site 2 - https://maps.app.goo.gl/WaxCQXhtcBC7VSui8

Located between Edgewater Ave and Beach St within the eroded cove. This proposed site will install a living shoreline with Marine sand fill and native plantings to fill and plant the eroded cove.

Site 2B - https://maps.app.goo.gl/WhLYMDwAaiZtnc2d8

Located on the corner of Edgewater Ave and Riverside Dr. This site will restore worth point revetment. Runyon Creek is also proposed as a “No Wake Zone”

Plan Sheet Overview.

1 - Title Sheet

- Overview of the Site locations

1A - General Notes

- General Construction Notes

- Notes are still being revised

1B  - Conventional Symbols

- Standard Symbology

1C – Overall Existing Conditions

- Existing Conditions of the project study area

- Provided by Survey

1D – Survey by Stewart, INC

2 – Summary of Quantities

- Quantities of materials to be produced at a later stage

G&EC-1 – Grading & Erosion Control Plan

- Stie 1 Grading and Erosion Control Plan

- Shows plan to fill to a 10:1 slope with Marine Sand

- Install Coir Fiber log to hold sand in Living Shoreline area

- Place Turbidity Curtain for sediment capture

G&EC-2 – Grading & Erosion Control Plan

- Stie 2 Grading and Erosion Control Plan

- Shows plan to fill to a 8:1 slope with Marine Sand

- Install Coir Fiber log to hold sand in Living Shoreline area

- Place Turbidity Curtain for sediment capture

- Remove existing groin in LSL area

- Restore brick groin with removed groin

G&EC-3 – Erosion Control Notes

- General Erosion Control Notes

- Note still being revised

G&EC-4 – Erosion Control Detail

- Details for Turbidity Curtain, Protection fence and Coir Fiber Log

D1 – Stie Plan

- Site 1

- Living Shoreline placement

D2 – Site Plan

- Site 2

- Living Shoreline placement

D3 – Site plan

- Worthy Point Restoration

- No Wake Zone

D4 – Detail Sheet

- Living Shoreline Wave-slope Detail

- Revetment rock plating detail

L1 – Planting Plan

- Site 1

- Planting area with plant

- Note: No Planting in Beach Access Zones

- Sagittaria lancifolia - Sagittaria lancifolia (Bulltongue Arrowhead, Duck Potato, Lance-leaved Arrowhead, Scythefruit Arrowhead) | North Carolina Extension Gardener Plant Toolbox

- Aristida stricta - Aristida stricta (Carolina Wiregrass, Pineland Threeawn, Pineland Three-awn, Wiregrass) | North Carolina Extension Gardener Plant Toolbox

L2 – Planting Plan

- Site 2

- Planting area with plants

L3 – Planting Details

- Herbaceous planting detail

- Waterfowl Exclusion Fence Detail